# Identify the Gap

Hydroficient IoT Cyber Defense Externship — Project 5, Step 1

Estimated time to complete: 15 minutes

Instructions: Complete all three parts below. Fill in the blanks, check the boxes that apply, and write your analysis in the spaces provided. Submit this document to the shared Google Doc when finished.

# Part 1: Review Your Project 4 Configuration

Open your mosquitto_tls.conf from Project 4 and answer these questions.

1. What port is the TLS listener using?

Port: 8883

2. What files are specified for the broker’s identity?

CA file: cafile certs/ca.pem

Certificate file: certfile certs/server.pem

Key file: keyfile certs/server-key.pem

3. Is there a require_certificate line?

☐  Yes, it’s set to true

☐  Yes, it’s set to false

- No, this line doesn’t exist

4. Based on your configuration, what type of TLS is this?

- One-Way TLS (server proves identity only)

☐  Mutual TLS (both sides prove identity)

# Part 2: The Rogue Device Scenario

Scenario: An attacker gains physical access to The Grand Marina’s network closet. They connect a Raspberry Pi to the network and try to connect to your MQTT broker.

Answer these questions about your current (Project 4) configuration:

1. Can the attacker’s device connect to the broker?

- Yes

☐  No

Explain why:

The attacker ‘s device can connect to the brokers because  the client certificate is not required and it allows connections without a username/ password which would mean the attacker can connect with verifying identity.

2. What would the attacker need to successfully connect?

☐  Just network access

- Network access + the CA certificate

☐  Network access + CA certificate + device certificate + private key

3. Once connected, what could the attacker do? (Check all that apply)

- Subscribe to all topics and read sensor data

- Publish fake sensor readings

- Send commands to HYDROLOGIC devices

- Pretend to be any device ID they want

- Take over legitimate device connections

# Part 3: Three Scenarios Where One-Way TLS Fails

List three realistic scenarios where your current one-way TLS configuration would fail to prevent an attack.

Hint: Consider insider threats, compromised devices, rogue hardware, man-in-the-middle attacks, or credential theft.

## Scenario 1

Scenario name:   Unauthorized Broker on the network

Describe the attack:

The attacker gains access to the hotel network (via guest Wi-Fi, a compromised device, or physical access) sets up a fake MQTT broker with a valid-looking certificate. Because one-way TLS only requires the server to prove its identity, sensors connecting to the network can be redirected to the rogue broker. The attacker intercepts all sensor readings and enters false data  for example reporting normal pressure while a real fault exists.

Why one-way TLS doesn’t stop it:

One-way TLS authenticates the server to the client, but not the client to the server. The sensors have no way to verify they are talking/ connecting to a legitimate broker. If the DNS is spoofed or the networking routing is compromised, the sensors can connect to the fake broker without warning.

## Scenario 2

Scenario name: Compromised Sensor Device

Describe the attack:

The maintenance contractor accesses the water sensor in the mechanical room and replaces it with a compromised device or extracts the stored credentials. The attacker has access to the certificate and can connect to the real broker as a trusted device publishing false readings and eavesdropping all traffic from other sensors on the same topics

Why one-way TLS doesn’t stop it:

TLS encrypts the channel but does not protect against an authorized device being used maliciously. Once the valid certificate gets into the wrong hands, the system cannot differentiate the attacker from a legitimate sensor. Without revoking the certificate and per device identity management, the stolen credentials grants access

## Scenario 3

Scenario name: Bypass Verification

Describe the attack:

The hotel IT employee who’s familiar with the system and knows the system is configured with verity=False writes a script that connects to the broker without presenting the certificate at all. They subscribe to all the sensor topics and discreetly log pressure and flow data over weeks. They map building activity patterns like which floors and occupied , when maintenance runs and when the building is the least active.

Why one-way TLS doesn’t stop it:

Connections with no certificates were accepted which means TLS is installed but not enforced. An insider who knows that detail can bypass authentication entirely. Encryption protects data in transit from outsiders but if the broker accepts unauthenticated connections, an insider on the network can connect and act like a trusted device. To close this gap, one would have to enforce require_certificate=True